Technical Report

Project #2: Automation of Concrete Mix Design

Prepared for:

Nebraska Department of Transportation (NDOT)

Prepared by:

Group K: Mina Awad, Nick Meier, Owen Betts

Department of Civil and Environmental Engineering

University of Nebraska-Omaha

CIVE 202: Civil Engineering Analysis II

February 24, 2026

Table of Contents

1. Introduction	3

2. Methods	3

2.1 Excel Logic Analysis	3

2.2 Python Function Development	3

2.3 Input Parameters	4

2.4 Output Format	5

2.5 Verification and Validation	5

3. Results and Discussion	6

3.1 Mix Scenario Descriptions	6

Scenario 1: NDOT Class 47B — Standard Highway Pavement	6

Scenario 2: NDOT Class 47BR — Bridge Deck Mix	6

Scenario 3: Fly Ash Replacement — Sustainable Pavement	6

Scenario 4: High-Performance Mix with Silica Fume	6

3.2 Scenario Results Comparison	6

3.3 Key Input Parameter Differences	7

3.4 Discussion	7

4. References	9

Table of Tables

Table 1: Excel formula to Python function mapping for all calculated variables.	4

Table 2: Summary of 16 input parameters collected via sequential user prompts	5

Table 3: Weight chart comparison for all four mix design scenarios (lb/yd³).	7

Table 4: Key input parameters across the four mix design scenarios.	7

# 1. Introduction

The Nebraska Department of Transportation (NDOT) has contracted our engineering team to migrate its existing concrete mix design model from an Excel-based workflow to an automated Python-based system. Concrete mix design is a fundamental process in highway and infrastructure construction, requiring precise proportioning of cement, water, aggregates, and supplementary cementitious materials (SCMs) to achieve specified strength, durability, and workability targets (ACI Committee 211, 2009). NDOT currently relies on a Mix Design Excel spreadsheet to perform these proportioning calculations, but the spreadsheet format presents limitations in terms of transparency, repeatability, and potential for future integration into web-based applications.

The NDOT Concrete Mix Design Excel workbook contains a series of interlinked formulas that convert user-provided material properties and design parameters into a final weight chart for one cubic yard of concrete (NDOT, 2024). While functional, the Excel-based approach is susceptible to manual input errors, lacks version control, and does not readily support automated scenario testing. By translating these calculations into Python, the logic becomes fixed, transparent, and repeatable, making it suitable for quality control and future development (NDOT, 2024b).

The objectives of this project are to: (1) translate all calculations from the Mix Design tab of the NDOT Excel workbook into clearly defined Python functions; (2) implement sequential user input prompts that collect design parameters in a logical order mirroring the Excel worksheet; (3) generate a clearly formatted weight chart for one cubic yard of concrete; and (4) research, prepare, and evaluate four realistic concrete mix design scenarios as a form of verification and validation of the automated model.

# 2. Methods

## 2.1 Excel Logic Analysis

The NDOT Concrete Mix Design Excel workbook was downloaded from the NDOT website and thoroughly analyzed (NDOT, 2024). The Mix Design tab contains shaded input cells where users enter 16 design parameters, identified as variables A through P in the Excel formulas. Every formula in the worksheet was documented, and the calculation dependencies between cells were traced to establish the correct sequential order for Python implementation. The calculation chain proceeds as follows: user inputs (A–P) feed into water weight (Q), which along with specific gravities determines cementitious volumes (R, S, T, U), then air and water volumes (V, W), total aggregate volume (X), and finally individual aggregate weights (Y, Z, AA).

## 2.2 Python Function Development

Each major calculation in the NDOT Excel worksheet was translated into a modular Python function with descriptive parameter names, docstrings, and return values. Two global constants were defined at the top of the program: CUBIC_YARD_FT3 = 27.0 (ft³ per cubic yard) and UNIT_WEIGHT_WATER = 62.4 (lb/ft³). These constants are referenced by the NDOT Excel formulas and are used throughout the calculation functions.

A total of 11 calculation functions and 2 utility functions (output formatting and calculation engine) were developed. Table 1 summarizes the Excel-to-Python translation for each calculated variable:

Table 1: Excel formula to Python function mapping for all calculated variables.

| Variable | Excel Formula | Python Function | Description |
| --- | --- | --- | --- |
| Q | (A+B+C+D) × E | calculate_water_weight_Q() | Water weight (lb/yd³) |
| R | A / (J × 62.4) | calculate_cement_volume_R() | Cement volume (ft³) |
| S | B / (K × 62.4) | calculate_fly_ash_volume_S() | Fly ash volume (ft³) |
| T | C / (L × 62.4) | calculate_silica_fume_volume_T() | Silica fume volume (ft³) |
| U | D / (M × 62.4) | calculate_other_scm_volume_U() | Other SCM volume (ft³) |
| V | (F/100) × 27 | calculate_air_volume_V() | Air volume (ft³) |
| W | Q / 62.4 | calculate_water_volume_W() | Water volume (ft³) |
| X | 27 – R – S – T – U – V – W | calculate_total_aggregate_volume_X() | Total aggregate volume (ft³) |
| Y | 62.4 × (G/100) × N × X | calculate_fine_aggregate_weight_Y() | Fine aggregate weight (lb/yd³) |
| Z | 62.4 × (H/100) × O × X | calculate_coarse_aggregate_weight_Z() | Coarse aggregate weight (lb/yd³) |
| AA | 62.4 × (I/100) × P × X | calculate_other_aggregate_weight_AA() | Other aggregate weight (lb/yd³) |

## 2.3 Input Parameters

The Python program collects 16 input parameters via sequential input() prompts. Each prompt uses the float() function for type conversion and includes descriptive labels with units. The parameters are organized into four groups mirroring the NDOT Excel worksheet layout, as shown in Table 2.

Table 2: Summary of 16 input parameters collected via sequential user prompts

| ID | Parameter | Units | Group |
| --- | --- | --- | --- |
| A | Weight of cement | lb/yd³ | Cementitious weights |
| B | Weight of fly ash | lb/yd³ | Cementitious weights |
| C | Weight of silica fume | lb/yd³ | Cementitious weights |
| D | Weight of other SCM | lb/yd³ | Cementitious weights |
| E | Target water–cement ratio | — | Design parameters |
| F | Target air content | % | Design parameters |
| G | Fine aggregate percentage | % | Aggregate proportions |
| H | Coarse aggregate percentage | % | Aggregate proportions |
| I | Other aggregate percentage | % | Aggregate proportions |
| J | Specific gravity of cement | — | Specific gravities |
| K | Specific gravity of fly ash | — | Specific gravities |
| L | Specific gravity of silica fume | — | Specific gravities |
| M | Specific gravity of other SCM | — | Specific gravities |
| N | Specific gravity of fine aggregate | — | Specific gravities |
| O | Specific gravity of coarse aggregate | — | Specific gravities |
| P | Specific gravity of other aggregate | — | Specific gravities |

## 2.4 Output Format

The program produces a formatted weight chart for one cubic yard of concrete, listing all component weights (cement, fly ash, silica fume, other SCM, water, fine aggregate, coarse aggregate, and other aggregate) and the total weight. All values are formatted to 3 decimal places using Python f-strings to match the NDOT Excel output format (NDOT, 2024). A volume verification check is also performed to confirm that the sum of all component volumes equals 27.000 ft³.

## 2.5 Verification and Validation

Four realistic concrete mix design scenarios were researched and evaluated to verify the Python implementation. Each scenario was run through the automated program and compared against the expected volumetric balance. The verification criterion is that R + S + T + U + V + W + X must equal exactly 27.000 ft³ for every scenario, confirming that the absolute volume method is correctly implemented (ACI Committee 211, 2009). The four scenarios represent a range of concrete applications including standard highway pavement, bridge decks, fly ash replacement mixes, and high-performance structural concrete.

# 3. Results and Discussion

## 3.1 Mix Scenario Descriptions

Four concrete mix design scenarios were selected to demonstrate the versatility of the automated tool and to verify its accuracy across different input configurations. Each scenario is described below.

### Scenario 1: NDOT Class 47B — Standard Highway Pavement

This scenario represents the most common NDOT concrete application for highway pavement. It uses 600 lb/yd³ of straight Portland cement with no supplementary cementitious materials, a water-cement ratio of 0.42, and 6.0% entrained air for freeze-thaw durability in Nebraska’s climate. The aggregate proportions are 40% fine, 55% coarse, and 5% other. This mix meets the requirements of NDOT Standard Specifications Section 1002 for Class 47B concrete (NDOT, 2017).

### Scenario 2: NDOT Class 47BR — Bridge Deck Mix

The bridge deck mix uses a higher cement content of 658 lb/yd³ and a reduced water-cement ratio of 0.40 to produce a denser, less permeable concrete. Bridge decks in Nebraska are exposed to deicing salts during winter, making chloride resistance a critical performance requirement (NDOT, 2017). The aggregate proportions are adjusted to 42% fine, 53% coarse, and 5% other to accommodate the increased paste volume from the higher cement content.

### Scenario 3: Fly Ash Replacement — Sustainable Pavement

This scenario demonstrates a sustainable mix design where 20% of the Portland cement is replaced with Class F fly ash (480 lb cement + 120 lb fly ash = 600 lb total cementitious). Fly ash replacement reduces the heat of hydration, improves long-term strength gain, and decreases the carbon footprint of the concrete (ACI Committee 211, 2009). The water-cement ratio is slightly increased to 0.44, and air content is raised to 6.5% per NDOT guidelines for SCM replacement mixes.

### Scenario 4: High-Performance Mix with Silica Fume

The high-performance scenario uses a ternary blend of cement (650 lb), fly ash (50 lb), and silica fume (40 lb) with a low water-cement ratio of 0.35. This produces the densest paste matrix of all four scenarios, with a total cementitious content of 740 lb/yd³. Silica fume fills the microscopic voids between cement particles, significantly reducing permeability and increasing compressive strength. This type of mix is used for high-durability applications such as parking garage decks, bridge piers, and marine structures (ACI Committee 211, 2009; NDOT, 2024b).

## 3.2 Scenario Results Comparison

Table 3 presents the calculated weight chart results for all four scenarios. All values are reported in pounds per cubic yard (lb/yd³), formatted to 3 decimal places.

Table 3: Weight chart comparison for all four mix design scenarios (lb/yd³).

| Material (lb/yd³) | Scenario 1 | Scenario 2 | Scenario 3 | Scenario 4 |
| --- | --- | --- | --- | --- |
| Cement (A) | 600.000 | 658.000 | 480.000 | 650.000 |
| Fly Ash (B) | 0.000 | 0.000 | 120.000 | 50.000 |
| Silica Fume (C) | 0.000 | 0.000 | 0.000 | 40.000 |
| Other SCM (D) | 0.000 | 0.000 | 0.000 | 0.000 |
| Water (Q) | 252.000 | 263.200 | 264.000 | 259.000 |
| Fine Agg. (Y) | 1200.580 | 1227.899 | 1251.605 | 1086.214 |
| Coarse Agg. (Z) | 1663.351 | 1561.275 | 1525.078 | 1641.711 |
| Other Agg. (AA) | 148.361 | 144.511 | 143.875 | 141.293 |
| TOTAL WEIGHT | 3864.292 | 3854.885 | 3784.558 | 3868.217 |

## 3.3 Key Input Parameter Differences

Table 4 summarizes the key input parameters that differ across the four scenarios. All scenarios use the same specific gravities for aggregates and cementitious materials to isolate the effects of proportioning changes.

Table 4: Key input parameters across the four mix design scenarios.

| Parameter | Scenario 1 | Scenario 2 | Scenario 3 | Scenario 4 |
| --- | --- | --- | --- | --- |
| Cement (lb/yd³) | 600 | 658 | 480 | 650 |
| Fly Ash (lb/yd³) | 0 | 0 | 120 | 50 |
| Silica Fume (lb/yd³) | 0 | 0 | 0 | 40 |
| W/C Ratio | 0.42 | 0.40 | 0.44 | 0.35 |
| Air Content (%) | 6.0 | 6.0 | 6.5 | 5.5 |
| Fine Agg. (%) | 40 | 42 | 43 | 38 |
| Coarse Agg. (%) | 55 | 53 | 52 | 57 |
| Total Cementitious | 600 | 658 | 600 | 740 |

## 3.4 Discussion

The results demonstrate several important relationships in concrete mix design that are consistent with established engineering principles.

Water-cement ratio and water demand: Scenario 4 has the lowest water demand at 259.000 lb/yd³ due to its low 0.35 w/c ratio, while Scenario 3 has the highest at 264.000 lb/yd³ with its 0.44 w/c ratio. Lower water-cement ratios produce stronger, more durable concrete but require careful attention to workability, which is typically maintained through the use of chemical admixtures (ACI Committee 211, 2009).

Effect of SCM replacement on paste volume: In Scenario 3, replacing 20% of the cement with fly ash maintains the same total cementitious weight (600 lb) but increases the paste volume because fly ash has a lower specific gravity (2.30) than cement (3.15). This means the same weight of fly ash occupies more volume, which reduces the space available for aggregates. The aggregate volume in Scenario 3 is therefore slightly lower than in Scenario 1, despite having the same total cementitious content.

High-performance ternary blends: Scenario 4 produces the highest total weight at 3868.217 lb/yd³ and the highest total cementitious content at 740 lb/yd³. The combination of low w/c ratio, silica fume, and fly ash creates a dense paste matrix with very low permeability. This type of mix is specified by NDOT for structures requiring long service life in aggressive environments (NDOT, 2017; NDOT, 2024b).

Volume verification: All four scenarios pass the critical verification check: the sum of all component volumes (R + S + T + U + V + W + X) equals exactly 27.000 ft³ in every case. This confirms that the Python implementation correctly replicates the absolute volume method used in the NDOT Excel worksheet and that the automated system produces consistent, repeatable results across diverse input configurations.

# 4. References

ACI Committee 211. (2009). Standard practice for selecting proportions for normal, heavyweight, and mass concrete (ACI 211.1-91). American Concrete Institute.

NDOT. (2017). Standard specifications for highway construction. Nebraska Department of Transportation. https://dot.nebraska.gov/media/g4qp4y0d/2017-specbook.pdf

NDOT. (2024). Concrete mix design submittal spreadsheet. Nebraska Department of Transportation. https://dot.nebraska.gov/media/jp3paote/mix-design-submittal.xlsx

NDOT. (2024b). Optimized aggregate gradations for Portland cement concrete mix designs evaluation. Nebraska Department of Transportation. https://dot.nebraska.gov/media/3isfdv45/final-report-p336.pdf